Innolite IL200 queries and answers.

Sr. 1:

Yes, you could have W axis( fast tool servo) on IL200, but no B axis any more, we decided not integrate B axis to IL200.

Sr.2

B axis is an option, you need to pay separately. The standard machine configuration may make some confusion, it is trying to say IL300 could be a 3 or 4 axis machine based on your option choice.

Sr.10

Yes, the Y axis is controlled by machine, but this is just for tool height adjustment, not for XYZC or XYZBC turning. Most of customers do not buy this option. Our standard tool tower has manual mechanical tool height adjustment function.

Sr.11,12

Sensor Module | Single means confocal sensor which could be used to measure sphere, aspheric surface and freeform surface.

Sensor Module | Double,  it is the combination of confocal sensor and tactile proble( Renishaw), usually they are used on center turning machine much more. The tactile probe could be used to measure the diameter of parts.

Sr.13

Yes, it is available for IL200. But LVDT probe is just used for sphere and asphere surface measurement( symmetrical part.) , it can not be used for freeform measurement.

Sr.15

The roughness measurement advanced system has more functions, it has white light interferometer function, confocal scanning function. and microscope function. ( Basically it is a Sensofar metrology instrument)

Sr.20

No. You need to pay separately for this active damping option. The standard one is passive damping system. Active damping system provide a little bit better performance.

Sr.23

Yes, it is compatible for all of Innolite machines.

Sr.24.

It is not mandatary, it is an option. It gives a little bit better performance for machine stability. The main function is to make the compressed air temperature much more stable.

Sr.26

Water/Air type could work independently, water/water type need extra chiller or cooling tower.

Water/Air type will makes hot air while Water/water does not make hot air.

Most of Chinese customers use water/air type.

Sr.48/sr49

Interface Disc w/sealing is an standard interface disc with sealing.

Interface cover is used to protect the nanogrip system from dusts.

Both of them are separate from standard interface disc. Usually you do not need them.

Sr.53

Yes, it is equipped with hard jaws.  And we do not sell hard jaws separately. It is ultra precision machine, so hard jaws do not worn fast.

Sr54.

No. it is not compatible with three jaw chuck. They are different.